# Expt. 1

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity fa is

Port ( a : in STD_LOGIC; b : in STD_LOGIC;

cin : in STD_LOGIC; s : out STD_LOGIC; co : out STD_LOGIC);

end fa;

architecture Behavioral of fa is begin

s <= a xor b xor cin;

co <= (a and b) or (b and cin) or (a and cin); end Behavioral;

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY fa_tb IS END fa_tb;

ARCHITECTURE behavior OF fa_tb IS

-- Component Declaration for the Unit Under Test (UUT) COMPONENT fa

PORT(

- : IN std_logic;

- : IN std_logic; cin : IN std_logic; s : OUT std_logic; co : OUT std_logic

);

END COMPONENT;

--Inputs

signal a : std_logic := '0'; signal b : std_logic := '0'; signal cin : std_logic := '0';

--Outputs signal s : std_logic;

signal co : std_logic;

-- No clocks detected in port list. Replace <clock> below with

-- appropriate port name BEGIN

-- Instantiate the Unit Under Test (UUT) uut: fa PORT MAP (

a => a,

b => b,

cin => cin, s => s,

co => co

);

-- Stimulus process stim_proc: process begin

a <= '0';

b <= '0';

cin <= '0'; wait for 20 ns;

a <= '0';

b <= '1';

cin <= '0'; wait for 20 ns;

a <= '1';

b <= '0';

cin <= '1'; wait for 20 ns;

a <= '1';

b <= '1';

cin <= '1'; wait for 20 ns;

end process; END;

=========================================================================

*	Final Report	*

=========================================================================

Final Results

RTL Top Level Output File Name	: fa.ngr Top Level Output File Name	: fa

Output Format	: NGC

Optimization Goal	: Speed

Keep Hierarchy	: No

Design Statistics

# IOs	: 5

Cell Usage :

# BELS	: 2

#	LUT3	: 2

# IO Buffers	: 5

#	IBUF	: 3

#	OBUF	: 2

=========================================================================

Device utilization summary:

Selected Device : 3s250epq208-5

| Number | of | Slices: | 1 | out | of | 2448 | 0% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | 4 input LUTs: | 2 | out | of | 4896 | 0% |
| Number | of | IOs: | 5 |  |  |  |  |
| Number | of | bonded IOBs: | 5 | out | of | 158 | 3% |

# Expt. 2

library IEEE;

use IEEE.STD_LOGIC_1164.ALL; use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity ALU_4bit is

Port ( a : in STD_LOGIC_VECTOR (3 downto 0); b : in STD_LOGIC_VECTOR (3 downto 0); sel : in STD_LOGIC_VECTOR (2 downto 0); y : out STD_LOGIC_VECTOR (3 downto 0));

end ALU_4bit;

architecture Behavioral of ALU_4bit is begin

process (a, b, sel) begin

case sel is

when "000" => y <= a + b; when "001" => y <= a - b; when "010" => y <= b - a; when "011" => y <= a and b; when "100" => y <= a or b; when "101" => y <= a nand b; when "110" => y <= a xor b; when "111" => y <= a nor b; when others => null;

end case; end process;

end Behavioral;

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY ALU_TB IS END ALU_TB;

ARCHITECTURE behavior OF ALU_TB IS

-- Component Declaration for the Unit Under Test (UUT) COMPONENT ALU_4bit

PORT(

- : IN std_logic_vector(3 downto 0);

- : IN std_logic_vector(3 downto 0); sel : IN std_logic_vector(2 downto 0); y : OUT std_logic_vector(3 downto 0)

);

END COMPONENT;

--Inputs

signal a : std_logic_vector(3 downto 0) := (others => '0'); signal b : std_logic_vector(3 downto 0) := (others => '0'); signal sel : std_logic_vector(2 downto 0) := (others => '0');

--Outputs

signal y : std_logic_vector(3 downto 0);

-- No clocks detected in port list. Replace <clock> below with

-- appropriate port name BEGIN

-- Instantiate the Unit Under Test (UUT) uut: ALU_4bit PORT MAP (

a => a,

b => b,

sel => sel, y => y

);

-- Stimulus process stim_proc: process begin

a <= "1010";

b <= "0101";

sel <= "000";

wait for 10 ns; sel <= "001";

wait for 10 ns; sel <= "010";

wait for 10 ns; sel <= "011";

wait for 10 ns; sel <= "100";

wait for 10 ns; sel <= "101";

wait for 10 ns; sel <= "110";

wait for 10 ns; sel <= "111";

wait for 10 ns;

end process; END;

=========================================================================

*	Final Report	*

=========================================================================

Final Results

RTL Top Level Output File Name	: ALU_4bit.ngr Top Level Output File Name	: ALU_4bit

Output Format	: NGC

Optimization Goal	: Speed

Keep Hierarchy	: No

Design Statistics

# IOs	: 15

Cell Usage :

# BELS	: 30

#	INV	: 1

#	LUT2	: 2

#	LUT3	: 2

#	LUT4	: 18

#	MUXF5	: 7

# IO Buffers	: 15

#	IBUF	: 11

#	OBUF	: 4

=========================================================================

Device utilization summary:

Selected Device : 3s250epq208-5

| Number | of | Slices: | 13 | out | of | 2448 | 0% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | 4 input LUTs: | 23 | out | of | 4896 | 0% |
| Number | of | IOs: | 15 |  |  |  |  |
| Number | of | bonded IOBs: | 15 | out | of | 158 | 9% |

Partition Resource Summary:

No Partitions were found in this design.

=========================================================================

# Expt. 3

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity shiftregister is

Port( clk,rst : in std_logic;

s : in std_logic_vector(3 downto 0);

m : in std_logic_vector(1 downto 0); -- mode y : out std_logic_vector(3 downto 0) );

end shiftregister;

architecture Behavioral of shiftregister is signal t : std_logic_vector(3 downto 0);

begin

process(clk, rst) begin

if rst = '1' then

y <= "0000";

elsif clk'event and clk = '1' then case m is

when "00" => -- SISO

t(3) <= s(0); -- LSB as input S(0) t(2) <= t(3);

t(1) <= t(2);

t(0) <= t(1);

y(0) <= t(0); -- LSB as output Y(0)

when "01" => -- SIPO

t(3) <= s(0); -- LSB as input S(0) t(2) <= t(3);

t(1) <= t(2);

t(0) <= t(1);

y <= t; -- parallel output

when "10" => -- PIPO

t <= s; y <= t;

when "11" => -- PISO

t <= s; -- 4 bit input S & t <= s

t(2) <= t(3); -- MSB input as t(3) <= S(3)

S(3)

end if; end process;

t(1) <= t(2);

t(0) <= t(1);

y(3) <= t(0); -- MSB as output Y(3) <=

when others => NULL; end case;

end Behavioral;

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY usr2_tb IS END usr2_tb;

ARCHITECTURE behavior OF usr2_tb IS COMPONENT shiftregister

PORT(

clk : IN std_logic; rst : IN std_logic;

s : IN std_logic_vector(3 downto 0); m : IN std_logic_vector(1 downto 0); y : OUT std_logic_vector(3 downto 0)

);

END COMPONENT;

-- Inputs

signal clk : std_logic := '0'; signal rst : std_logic := '0';

signal s : std_logic_vector(3 downto 0) := (others => '0'); signal m : std_logic_vector(1 downto 0) := (others => '0');

-- Outputs

signal y : std_logic_vector(3 downto 0); BEGIN

-- Instantiate the Unit Under Test (UUT) uut: shiftregister PORT MAP (

clk => clk, rst => rst, s => s,

m => m, y => y

);

-- Clock process definitions clk_process : process

begin

end process;

clk <= '0'; wait for 3 ns; clk <= '1'; wait for 3 ns;

-- Stimulus process stim_proc: process begin

rst <= '1'; wait for 5 ns;

rst <= '0'; wait for 5 ns;

-- Test SISO Mode m <= "00";

s <= "1-0U";

wait for 50 ns;

-- Test SIPO Mode m <= "01";

s <= "1UXX";

wait for 50 ns;

-- Test PISO Mode m <= "11";

s <= "1111";

wait for 50 ns;

-- Test PIPO Mode m <= "10";

s <= "1UU0";

wait for 50 ns; end process;

END;

Output for SISO and SIPO

Output for PISO and PIPO

=========================================================================

*	Final Report	*

=========================================================================

Final Results

RTL Top Level Output File Name	: shiftregister.ngr Top Level Output File Name	: shiftregister Output Format	: NGC

Optimization Goal	: Speed

Keep Hierarchy	: No

Design Statistics

# IOs	: 12

Cell Usage :

# BELS	: 11

#	INV	: 1

#	LUT3	: 3

#	LUT3_L	: 2

#	LUT4	: 5

# FlipFlops/Latches	: 8

#	FDC	: 4

#	FDE	: 4

# Clock Buffers	: 1

#	BUFGP	: 1

# IO Buffers	: 11

#	IBUF	: 7

#	OBUF	: 4

=========================================================================

Device utilization summary:

‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐

Selected Device : 3s250epq208‐5

| Number | of | Slices: | 5 | out | of | 2448 | 0% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | Slice Flip Flops: | 8 | out | of | 4896 | 0% |
| Number | of | 4 input LUTs: | 11 | out | of | 4896 | 0% |
| Number | of | IOs: | 12 |  |  |  |  |
| Number | of | bonded IOBs: | 12 | out | of | 158 | 7% |
| Number | of | GCLKs: | 1 | out | of | 24 | 4% |

‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐

Partition Resource Summary:

‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐

No Partitions were found in this design.

‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐‐

# Expt. 4

-- Modulo-10 Counter with Synchronous Reset : 0 to 9 (0000 to 1001) library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_unsigned.all;

entity counter is

port ( CLK : in std_logic;

RST : in std_logic; -- Synchronous reset input RST active

high

end counter;

Q : out std_logic_vector(3 downto 0));

architecture counter_arch of counter is signal TEMP_Q : std_logic_vector(3 downto 0); begin

process(CLK) begin

if rising_edge(CLK) then -- if clk'event and clk = '1' then

if RST = '1' then -- active high and it is inside clock therfore its synchronous reset

TEMP_Q <= (others => '0'); -- TEMP_Q <= "0000"

else

if (TEMP_Q = "1001") then

TEMP_Q <= (others => '0'); -- TEMP_Q <= "0000"

else

TEMP_Q <= TEMP_Q + 1; -- up counting end if ;

end if ; end if;

end process; Q <= TEMP_Q;

end counter_arch;

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

ENTITY counter_tb IS END counter_tb;

ARCHITECTURE behavior OF counter_tb IS

-- Component Declaration for the Unit Under Test (UUT) COMPONENT counter

PORT(

CLK : IN std_logic; RST : IN std_logic;

Q : OUT std_logic_vector(3 downto 0)

);

END COMPONENT;

--Inputs

signal CLK : std_logic := '0'; signal RST : std_logic := '0';

--Outputs

signal Q : std_logic_vector(3 downto 0);

-- Clock period definitions BEGIN

-- Instantiate the Unit Under Test (UUT) uut: counter PORT MAP (

CLK => CLK, RST => RST, Q => Q

);

-- Clock process definitions CLK_process :process

begin

end process;

CLK <= '0';

wait for 5 ns; CLK <= '1';

wait for 5 ns;

-- Stimulus process stim_proc: process begin

-- hold reset state for 100 ns.

| RST | <= | '1'; |  |  |
| --- | --- | --- | --- | --- |
|  |  | wait | for | 20 ns; |
| RST | <= | '0'; |  |  |
|  |  | wait | for | 200 ns; |
| RST | <= | '1'; |  |  |
|  |  | wait | for | 10 ns; |
| RST | <= | '0'; |  |  |
|  |  | wait | for | 300 ns; |

end process; END;

=========================================================================

*	Final Report	*

=========================================================================

Final Results

RTL Top Level Output File Name	: counter.ngr Top Level Output File Name	: counter

Output Format	: NGC

Optimization Goal	: Speed

Keep Hierarchy	: No

Design Statistics

# IOs	: 6

Cell Usage :

# BELS	: 6

#	INV	: 1

#	LUT2	: 1

#	LUT2_L	: 1

#	LUT3	: 1

#	LUT4	: 2

# FlipFlops/Latches	: 4

#	FDR	: 4

# Clock Buffers	: 1

#	BUFGP	: 1

# IO Buffers	: 5

#	IBUF	: 1

#	OBUF	: 4

=========================================================================

Device utilization summary:

Selected Device : 3s250epq208-5

| Number | of | Slices: | 3 | out | of | 2448 | 0% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | Slice Flip Flops: | 4 | out | of | 4896 | 0% |
| Number | of | 4 input LUTs: | 6 | out | of | 4896 | 0% |
| Number | of | IOs: | 6 |  |  |  |  |
| Number | of | bonded IOBs: | 6 | out | of | 158 | 3% |
| Number | of | GCLKs: | 1 | out | of | 24 | 4% |

# Expt. 5

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_unsigned.ALL; use IEEE.STD_LOGIC_arith.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

--use IEEE.NUMERIC_STD.ALL;

-- Uncomment the following library declaration if instantiating

-- any Xilinx primitives in this code.

--library UNISIM;

--use UNISIM.VComponents.all;

entity FIFO_4bit is

Port ( clk,rst : in STD_LOGIC; enr,enw : in STD_LOGIC;

datain : in STD_LOGIC_VECTOR (3 downto 0); dataout : out STD_LOGIC_VECTOR (3 downto 0); full,empty : out STD_LOGIC);

end FIFO_4bit;

architecture Behavioral of FIFO_4bit is

type memory_type is array (0 to 7) of std_logic_vector(3 downto 0); signal memory : memory_type;

signal readptr,writeptr : std_logic_vector(3 downto 0):="0000";

--signal cnt:std_logic_vector(24 downto 0);

--signal clk1:std_logic;

begin

FIFO_Process: process(clk)

begin

if(clk'event and clk='1' and enw ='1' and enr='0') then --WRITE

OPERATION

memory(conv_integer(writeptr)) <= datain; writeptr <= writeptr + '1';

end if;

if(clk'event and clk='1' and enr ='1' and enw='0') then --READ

OPERATION

dataout <= memory(conv_integer(readptr));

readptr <= readptr + '1';

end if;

if(readptr = "1000") then

empty<='1';

else

empty<='0';

end if;

if(writeptr = "1000") then

full <='1';

else

full <='0';

end if; end process;

end Behavioral;

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

--USE ieee.numeric_std.ALL;

ENTITY test1 IS END test1;

ARCHITECTURE behavior OF test1 IS

-- Component Declaration for the Unit Under Test (UUT) COMPONENT FIFO_4bit

PORT(

clk : IN std_logic; rst : IN std_logic; enr : IN std_logic; enw : IN std_logic;

datain : IN std_logic_vector(3 downto 0); dataout : OUT std_logic_vector(3 downto 0); full : OUT std_logic;

empty : OUT std_logic

);

END COMPONENT;

--Inputs

signal clk : std_logic := '0'; signal rst : std_logic := '0'; signal enr : std_logic := '0'; signal enw : std_logic := '0';

signal datain : std_logic_vector(3 downto 0) := (others => '0');

--Outputs

signal dataout : std_logic_vector(3 downto 0); signal full : std_logic;

signal empty : std_logic;

-- Clock period definitions

constant clk_period : time := 10 ns; BEGIN

-- Instantiate the Unit Under Test (UUT) uut: FIFO_4bit PORT MAP (

clk => clk, rst => rst, enr => enr,

enw => enw, datain => datain,

dataout => dataout, full => full,

empty => empty

);

-- Clock process definitions clk_process :process

begin

end process;

clk <= '1'; wait for 5 ns; clk <= '0'; wait for 5 ns;

-- Stimulus process stim_proc: process begin

enw<='1';

enr<='0'; datain<="0001"; wait for 10ns;

datain<="0011"; wait for 10ns;

datain<="0111"; wait for 10ns;

datain<="1111"; wait for 10ns;

datain<="1110"; wait for 10ns;

datain<="1100"; wait for 10ns;

datain<="1000"; wait for 10ns;

datain<="0000"; wait for 10ns;

enw<='0';

enr<='1';

wait for 80ns;

end process; END;

=========================================================================

*	Final Report	*

=========================================================================

Final Results

RTL Top Level Output File Name	: FIFO_4bit.ngr Top Level Output File Name	: FIFO_4bit Output Format	: NGC

Optimization Goal	: Speed

Keep Hierarchy	: No

Design Statistics

# IOs	: 14

Cell Usage :

# BELS	: 13

#	GND	: 1

#	INV	: 2

#	LUT2	: 4

#	LUT3	: 2

#	LUT4	: 4

# FlipFlops/Latches	: 12

#	FDE	: 12

# RAMS	: 4

#	RAM16X1D	: 4

# Clock Buffers	: 1

#	BUFGP	: 1

# IO Buffers	: 12

#	IBUF	: 6

#	OBUF	: 6

=========================================================================

Device utilization summary:

Selected Device : 3s250epq208-5

| Number | of | Slices: | 10 | out | of | 2448 | 0% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | Slice Flip Flops: | 12 | out | of | 4896 | 0% |
| Number | of | 4 input LUTs: | 20 | out | of | 4896 | 0% |

Number used as logic:	12

Number used as RAMs:	8

| Number | of | IOs: |  | 14 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | bonded | IOBs: | 13 | out | of | 158 | 8% |
| Number | of | GCLKs: |  | 1 | out | of | 24 | 4% |

# Expt. 6

library IEEE;

use IEEE.STD_LOGIC_1164.ALL; use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity lcd is

Port ( clk,reset : in std_logic; RS,EN : out std_logic;

data : out std_logic_vector(7 downto 0));

end lcd;

architecture Behavioral of lcd is

type state_type is (s0,s1,s2,s3,s4,s5,s6,s7,s8,s9,s10,s11, s12,s13,s14,s15,s16,s17,s18,s19,s20,s21,s22,s23);

signal state:state_type; begin

process(clk, reset, state) begin

if reset = '1' then

state <= s0;

elsif rising_edge(clk) then if state = s0 then

state <= s1;

RS<='0'; -- Write commands to LCD. EN <= '1';

data <= "00110000"; -- Function set for 8 bit interface, 1 line mode and 5x7 dot matrix.

end if;

if state = s1 then

state <= s2; EN <= '0';

ON.

end if;

if state = s2 then

state <= s3; EN <= '1';

data <= "00001111"; -- Display cursor and blinking

end if;

if state = s3 then

state <= s4; EN <= '0';

end if;

if state = s4 then

state <= s5; EN <= '1';

data <= "00000001";	-- Clear display.

end if;

if state = s5 then

state <= s6; EN <= '0';

end if;

if state = s6 then

end if;

state <= s7; EN <= '1';

data <= "10000100"; -- Display address.

if state = s7 then

state <= s8; EN <= '0';

end if;

if state = s8 then

RS <= '1'; -- Write data to LCD. state <= s9;

EN <= '1';

data <= "01010111"; --W

end if;

if state = s9 then

state <= s10; EN <= '0';

end if;

if state = s10 then

state <= s11; EN <= '1';

data <= "01000101"; --E

end if;

if state = s11 then

state <= s12; EN <= '0';

end if;

if state = s12 then

state <= s13; EN <= '1';

data <= "01001100"; --L

end if;

if state = s13 then

state <= s14; EN <= '0';

end if;

if state = s14 then

state <= s15; EN <= '1';

data <= "10110000"; --(-)

end if;

if state = s15 then

state <= s16; EN <= '0';

end if;

if state = s16 then

state <= s17; EN <= '1';

data <= "01000011"; --C

end if;

if state = s17 then

state <= s18; EN <= '0';

end if;

if state = s18 then

state <= s19; EN <= '1';

data <= "01001111"; --O

end if;

if state = s19 then

state <= s20; EN <= '0';

end if;

if state = s20 then

state <= s21; EN <= '1';

data <= "01001101"; --M

end if;

if state = s21 then

state <= s22; EN <= '0';

end if;

if state = s22 then

state <= s23; EN <= '1';

data <= "01000101"; --E

end if;

if state = s23 then

EN <= '0';

end if;

end if; end process;

end Behavioral;

=========================================================================

*	Final Report	*

=========================================================================

Final Results

RTL Top Level Output File Name	: lcd.ngr Top Level Output File Name	: lcd

Output Format	: NGC

Optimization Goal	: Speed

Keep Hierarchy	: No

Design Statistics

# IOs	: 12

Cell Usage :

# BELS	: 30

#	GND	: 1

#	INV	: 1

#	LUT2_L	: 2

#	LUT3	: 1

#	LUT3_D	: 1

#	LUT3_L	: 3

#	LUT4	: 17

#	LUT4_L	: 3

#	VCC	: 1

# FlipFlops/Latches	: 34

#	FDC	: 22

#	FDCE	: 1

#	FDE	: 10

#	FDP	: 1

# Clock Buffers	: 1

#	BUFGP	: 1

# IO Buffers	: 11

#	IBUF	: 1

#	OBUF	: 10

=========================================================================

Device utilization summary:

Selected Device : 3s250epq208-5

| Number | of | Slices: | 24 | out | of | 2448 | 0% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number | of | Slice Flip Flops: | 34 | out | of | 4896 | 0% |
| Number | of | 4 input LUTs: | 28 | out | of | 4896 | 0% |
| Number | of | IOs: | 12 |  |  |  |  |
| Number | of | bonded IOBs: | 12 | out | of | 158 | 7% |
| Number | of | GCLKs: | 1 | out | of | 24 | 4% |